IMPLEMENTATION GUIDE

# Connecting the Romeson LED8
Customer Display to Odoo POS

Installation, configuration, testing and troubleshooting for Bahdela POS terminals

| Hardware | Romeson eight-digit LED customer display |
| --- | --- |
| Serial settings | COM2 ¦ 2400 baud ¦ 8 data bits ¦ no parity ¦ 1 stop bit |
| Supported POS sites | stage.bahdela.or.tz ¦ erp.bahdela.co.tz ¦ *.odoo.com |
| Connector package | Romeson-Odoo-LED8 v1.0.1 |

Purpose: This guide explains how the Odoo POS total shown in Microsoft Edge is sent to the Romeson rear LED display automatically. The integration does not require changing the Odoo server.

# 1. How the integration works

The Romeson display is a serial device. Odoo runs as a web application inside Microsoft Edge, and web pages cannot write directly to Windows COM ports. The connector therefore uses two local components:

| 1 Odoo POS Shows the current order total | 2 Edge extension Reads only the visible numeric total | 3 Windows bridge Receives the amount locally | 4 COM2 display Shows the amount at 2400 baud |
| --- | --- | --- | --- |

Data privacy: Only the numeric amount is sent to 127.0.0.1 on the same computer. Product names, customer details, passwords and payment information are not transmitted by the connector.

# 2. What was confirmed before integration

The display was tested from Windows Command Prompt before connecting it to Odoo. This separated the hardware problem from the browser integration problem.

## 2.1 Correct serial port

COM1 produced no useful display response. COM2 changed the Romeson screen, confirming that COM2 is the connected display port.

## 2.2 Correct communication speed

At 9600 baud, the display showed corrupted symbols. The LED8 display uses 2400 baud after power-on. Changing COM2 to 2400 baud fixed the character corruption.

mode COM2 BAUD=2400 PARITY=N DATA=8 STOP=1

## 2.3 Working LED8 command

The following PowerShell test initializes the display, clears it, selects the Total indicator and sends 25000.00:

powershell -Command "$p=New-Object System.IO.Ports.SerialPort 'COM2',2400,'None',8,'One'; $p.Open(); $b=[byte[]](@(27,64,12,27,115,50)+[Text.Encoding]::ASCII.GetBytes('25000.00')); $p.Write($b,0,$b.Length); Start-Sleep -Seconds 3; $p.Close()"

Expected result: The Romeson display shows 25000.00 without corrupted symbols. This confirms the port, speed and command protocol.

# 3. Package contents

| File or folder | Purpose |
| --- | --- |
| install.bat | Copies the connector, reserves the local address and creates automatic startup. |
| bridge\romeson-bridge.ps1 | Writes LED8 commands to COM2 at 2400 baud. |
| extension\ | Microsoft Edge extension that reads the Odoo POS total. |
| test-display.bat | Displays 25000.00 to verify the local bridge. |
| start-bridge.bat / stop-bridge.bat | Starts or stops the Windows bridge manually. |
| uninstall.bat | Stops automatic startup and removes the local address reservation. |

# 4. Installation procedure

1. Extract the ZIP package

Extract Romeson-Odoo-LED8-v1.0.1.zip to a normal folder. Do not run install.bat from inside the ZIP preview window.

2. Run the installer as administrator

Right-click install.bat and select Run as administrator. Administrator rights are needed to reserve the local bridge address.

3. Open Edge extensions

In Microsoft Edge, type the following address and press Enter:

edge://extensions

4. Enable Developer mode

Turn on Developer mode using the switch on the Extensions page.

5. Load the extension

Click Load unpacked. In the folder selector, paste the path below and select the extension folder:

%LOCALAPPDATA%\RomesonOdooBridge\extension

6. Pin the extension

Open the Edge Extensions menu and pin Odoo POS to Romeson LED8 so its status and test buttons remain easy to access.

7. Run the display test

Click the extension icon and select Test 25,000.00. The extension should report Connected: COM2 at 2400 baud, and the Romeson display should show 25000.00.

8. Open Odoo POS

Open one of the supported POS pages, enter the POS session and add a product. The rear display should update when the order total changes.

## Supported Odoo POS addresses

https://stage.bahdela.or.tz/pos/ui/*

https://erp.bahdela.co.tz/pos/ui/*

https://*.odoo.com/pos/ui/*

Important: The extension activates on /pos/ui/ pages. Opening only the website home page does not activate the POS reader; enter an actual Point of Sale session.

# 5. Testing and acceptance checks

Run test-display.bat and confirm that 25000.00 appears.

Open the extension popup and confirm COM2 and 2400 baud are reported.

Add one product in Odoo POS and confirm the rear display changes.

Change quantity and confirm the displayed total updates.

Remove the product or cancel the order and confirm the display returns to zero or clears.

Restart Windows and confirm the bridge starts automatically.

Test both stage.bahdela.or.tz and erp.bahdela.co.tz if both systems are used on the terminal.

# 6. Troubleshooting

| Problem | Likely cause | Action |
| --- | --- | --- |
| Extension says Not connected | Windows bridge is not running | Run start-bridge.bat, then reopen the extension popup. |
| Access to COM2 is denied | Another program has opened COM2 | Close other POS/display utilities and restart the bridge. |
| Corrupted symbols | Wrong baud rate | Restart the display and confirm the bridge uses 2400 baud. |
| CLI test works but Odoo does not | Extension not loaded, wrong page, or page layout changed | Reload the extension, open a /pos/ui/ page, refresh POS and inspect the extension status. |
| Works until Windows restarts | Startup shortcut missing | Run install.bat as administrator again. |
| COM2 is missing | Serial device or BIOS port disabled | Check Device Manager and confirm Communications Port (COM2) is enabled. |

## Diagnostic addresses and files

http://127.0.0.1:8765/health

%LOCALAPPDATA%\RomesonOdooBridge\bridge.log

# 7. Updating the extension

When a new package version is supplied, use this controlled update process:

Extract the new ZIP file.

Run the new install.bat as administrator.

Open edge://extensions.

Remove the old Odoo POS to Romeson LED8 extension.

Click Load unpacked and select %LOCALAPPDATA%\RomesonOdooBridge\extension.

Reload the Odoo POS page and perform the 25000.00 test.

# 8. Uninstallation

Run uninstall.bat, remove the extension from edge://extensions, and then delete the following folder if the connector is no longer required:

%LOCALAPPDATA%\RomesonOdooBridge

# 9. Technical summary

| Serial device | COM2 |
| --- | --- |
| Communication | 2400 baud, 8-N-1, no flow control |
| Display protocol | LED8 / ESC-POS-compatible byte commands |
| Local bridge | http://127.0.0.1:8765 |
| Browser | Microsoft Edge, Manifest V3 extension |
| Server changes | None; works with hosted Odoo POS pages |

Final result: The Romeson customer display detects the active Odoo POS order and presents the numeric total to the customer through COM2, without installing a custom module on the Odoo server.